# Inter-VLAN Routing Exercise (Router-on-a-stick)

Topology:

The local network is separated into VLANs according to the following table:

| VLAN ID | VLAN Name | Ports |
| --- | --- | --- |
| 10 | IT | F0/6-12 |
| 20 | HR | F0/13-17 |
| 30 | Sales | F0/18-24 |
| 90 | Management | F0/1-5 |

- Using VLSM, reproduce and fill in the following table:
Knowing that:

  - The network address is: 10.0.0.0/8

| Site | @Network | Mask | 1st Address | Last Address | Broadcast Address |
| --- | --- | --- | --- | --- | --- |
| IT (100h) | 10.0.0.0 | 255.255.255.128 | 10.0.0.1 | 10.0.0.126 | 10.0.0.127 |
| HR (59h) | 10.0.0.128 | 255.255.255.192 | 10.0.0.129 | 10.0.0.190 | 10.0.0.191 |
| Sales (27h) | 10.0.0.192 | 255.255.255.224 | 10.0.0.193 | 10.0.0.222 | 10.0.0.223 |
| Management (12h) | 10.0.0.224 | 255.255.255.240 | 10.0.0.225 | 10.0.0.238 | 10.0.0.239 |

- Create and name the VLANs on the switches as shown in the mockup above.

- Configure the ports in Access mode.

- Configure the trunk ports (NB: native VLAN 90).

- Configure inter-VLAN routing on router R1.

- Configure the management interface on the switches.

- Configure remote access via Telnet on the switches and router R1

# Verification:

Test connectivity, display the routing table on R1, and from the Admin laptop test remote access.

P r o f e s s o 
      r  Y o u s s e f	P a g e 1 | 1